// FiberGratingAnalyzer.cpp

#include "FiberGratingAnalyzer.h"

#include <boost/asio.hpp>

#include <thread>

#include <memory>

#include <iostream>

#include <cstring>

#include <iomanip>

#include <sstream>

// PIMPL实现类

class FiberGratingAnalyzer::Impl {

public:

Impl() : io_context(), socket(io_context), receiving(false) {}

~Impl() {

stop();

}

bool setPort(int port) {

try {

boost::asio::ip::udp::endpoint endpoint(boost::asio::ip::udp::v4(), port);

socket.open(endpoint.protocol());

socket.bind(endpoint);

return true;

}

catch (const std::exception& e) {

std::cerr << "端口设置失败: " << e.what() << std::endl;

return false;

}

}

bool startReceiving(const DataCallback& callback) {

if (receiving) {

return false;

}

receiving = true;

receiveThread = std::thread([this, callback]() {

receiveData(callback);

});

return true;

}

void stop() {

receiving = false;

if (receiveThread.joinable()) {

receiveThread.join();

}

if (socket.is_open()) {

socket.close();

}

}

bool isReceiving() const {

return receiving;

}

private:

void receiveData(const DataCallback& callback) {

std::vector<uint8_t> buffer(2048); // 足够大的缓冲区

while (receiving) {

try {

boost::asio::ip::udp::endpoint remote_endpoint;

boost::system::error_code error;

size_t length = socket.receive_from(

boost::asio::buffer(buffer), remote_endpoint, 0, error);

if (error && error != boost::asio::error::message_size) {

if (receiving) { // 只有在仍然接收时才报告错误

std::cerr << "接收错误: " << error.message() << std::endl;

}

continue;

}

// 解析接收到的数据

auto sensorData = parseData(buffer.data(), length);

if (!sensorData.empty() && callback) {

callback(sensorData);

}

}

catch (const std::exception& e) {

if (receiving) { // 只有在仍然接收时才报告错误

std::cerr << "接收异常: " << e.what() << std::endl;

}

}

}

}

//std::vector<SensorData> parseData(const uint8_t* data, size_t length) {

//    std::vector<SensorData> result;

//    // 检查最小长度和数据头

//    if (length < 5) return result; // 至少需要5字节(4字节头+1字节通道数)

//    // 检查数据头 "FBGV" 或 "FBGW"

//    if (data[0] == 0x46 && data[1] == 0x42 && data[2] == 0x47 &&

//        (data[3] == 0x56 || data[3] == 0x57)) {

//        bool hasPhysicalValue = (data[3] == 0x56); // 'V'表示有物理量

//        uint8_t channelCount = data[4]; // 通道数

//        size_t pos = 5; // 当前位置

//        for (uint8_t ch = 0; ch < channelCount && pos < length; ch++) {

//            if (pos >= length) break;

//            uint8_t sensorCount = data[pos++]; // 当前通道的传感器数量

//            for (uint8_t s = 0; s < sensorCount && pos < length; s++) {

//                SensorData sensor;

//                sensor.channel = ch + 1; // 通道从1开始

//                sensor.sequence = s + 1; // 序列从1开始

//                sensor.hasPhysicalValue = hasPhysicalValue;

//                // 读取波长 (2字节，大端)

//                if (pos + 1 >= length) break;

//                uint16_t wavelengthRaw = (data[pos] << 8) | data[pos + 1];

//                pos += 2;

//                sensor.wavelength = wavelengthRaw / 1000.0 + 1520.0; // 转换为nm

//                // 如果有物理量，读取物理量 (4字节，大端)

//                if (hasPhysicalValue) {

//                    if (pos + 3 >= length) break;

//                    // 将4字节大端数据转换为float

//                    uint32_t physicalRaw = (data[pos] << 24) |

//                        (data[pos + 1] << 16) |

//                        (data[pos + 2] << 8) |

//                        data[pos + 3];

//                    pos += 4;

//                    // 重新解释字节为float

//                    sensor.physicalValue = *reinterpret_cast<float*>(&physicalRaw);

//                }

//                result.push_back(sensor);

//            }

//        }

//    }

//    return result;

//}

//std::vector<SensorData> parseData(const uint8_t* data, size_t length) {

//    std::vector<SensorData> result;

//    // 检查最小长度和数据头

//    if (length < 5) return result; // 至少需要5字节(4字节头+1字节通道数)

//    // 检查数据头 "FBGV" 或 "FBGW"

//    if (data[0] == 0x46 && data[1] == 0x42 && data[2] == 0x47 &&

//        (data[3] == 0x56 || data[3] == 0x57)) {

//        bool hasPhysicalValue = (data[3] == 0x56); // 'V'表示有物理量

//        uint8_t channelCount = data[4]; // 通道数

//        size_t pos = 5; // 当前位置

//        for (uint8_t ch = 0; ch < channelCount && pos < length; ch++) {

//            if (pos >= length) break;

//            uint8_t sensorCount = data[pos++]; // 当前通道的传感器数量

//            for (uint8_t s = 0; s < sensorCount && pos < length; s++) {

//                SensorData sensor;

//                sensor.channel = ch + 1; // 通道从1开始

//                sensor.sequence = s + 1; // 序列从1开始

//                sensor.hasPhysicalValue = hasPhysicalValue;

//                // 读取波长 (2字节，大端)

//                if (pos + 1 >= length) break;

//                uint16_t wavelengthRaw = (static_cast<uint32_t>(data[pos]) << 8) |

//                    (static_cast<uint32_t>(data[pos + 1])); // 添加类型转换避免潜在问题

//                pos += 2;

//                sensor.wavelength = wavelengthRaw / 1000.0 + 1520.0; // 转换为nm

//                // 如果有物理量，读取物理量 (4字节，大端)

//                if (hasPhysicalValue) {

//                    if (pos + 3 >= length) break;

//                    // 1. 将4字节大端数据组合成uint32_t

//                    uint32_t physicalRaw = (static_cast<uint32_t>(data[pos]) << 24) |

//                        (static_cast<uint32_t>(data[pos + 1]) << 16) |

//                        (static_cast<uint32_t>(data[pos + 2]) << 8) |

//                        (static_cast<uint32_t>(data[pos + 3]));

//                    pos += 4;

//                    // 2. 安全的方式：使用 memcpy 进行类型转换 [1,6,7](@ref)

//                    // 或者使用 C++20 的 std::bit_cast (如果编译器支持)

//                    std::memcpy(&sensor.physicalValue, &physicalRaw, sizeof(float));

//                    // 注意：如果数据流中的浮点数字节序与主机不同，此处可能需要额外的字节序转换

//                    // 例如，如果数据是大端序，而主机是小端序，则需要交换字节序

//                    // sensor.physicalValue = swapFloatEndian(sensor.physicalValue); // 需要自定义此函数

//                }

//                result.push_back(sensor);

//            }

//        }

//    }

//    return result;

//}

//std::vector<SensorData> parseData(const uint8_t* data, size_t length) {

//    std::vector<SensorData> result;

//

//    // 检查最小长度和数据头

//    if (length < 5) return result; // 至少需要5字节(4字节头+1字节通道数)

//

//    // 检查数据头 "FBGV" 或 "FBGW"

//    if (data[0] == 0x46 && data[1] == 0x42 && data[2] == 0x47 &&

//        (data[3] == 0x56 || data[3] == 0x57)) {

//

//        bool hasPhysicalValue = (data[3] == 0x56); // 'V'表示有物理量

//        uint8_t channelCount = data[4]; // 通道数

//

//        size_t pos = 5; // 当前位置

//        std::cout << "=== 开始解析数据包 ===" << std::endl;

//        std::cout << "数据头: FBG" << (char)data[3] << std::endl;

//        std::cout << "通道数: " << (int)channelCount << std::endl;

//        std::cout << "数据总长度: " << length << " 字节" << std::endl;

//

//        for (uint8_t ch = 0; ch < channelCount && pos < length; ch++) {

//            if (pos >= length) break;

//

//            uint8_t sensorCount = data[pos++]; // 当前通道的传感器数量

//            std::cout << "通道 " << (ch + 1) << " 传感器数量: " << (int)sensorCount << std::endl;

//

//            for (uint8_t s = 0; s < sensorCount && pos < length; s++) {

//                SensorData sensor;

//                sensor.channel = ch + 1; // 通道从1开始

//                sensor.sequence = s + 1; // 序列从1开始

//                sensor.hasPhysicalValue = hasPhysicalValue;

//

//                // 读取波长 (2字节，大端)

//                if (pos + 1 >= length) break;

//

//                // 保存原始字节用于调试输出

//                uint8_t byte1 = data[pos];

//                uint8_t byte2 = data[pos + 1];

//

//                uint16_t wavelengthRaw = (static_cast<uint32_t>(byte1) << 8) |

//                    (static_cast<uint32_t>(byte2));

//                pos += 2;

//                sensor.wavelength = wavelengthRaw / 1000.0 + 1520.0; // 转换为nm

//

//                // 打印波长原始数据详细信息[1,3](@ref)

//                std::cout << "通道 " << std::setw(2) << (int)sensor.channel

//                    << " 传感器 " << std::setw(2) << (int)sensor.sequence

//                    << " - 原始字节: 0x" << std::hex << std::uppercase

//                    << std::setw(2) << std::setfill('0') << (int)byte1

//                    << " " << std::setw(2) << std::setfill('0') << (int)byte2

//                    << std::dec << std::setfill(' ')  // 恢复十进制输出[1](@ref)

//                    << " | 原始值: " << std::setw(5) << wavelengthRaw

//                    << " | 计算波长: " << std::fixed << std::setprecision(3)

//                    << sensor.wavelength << " nm" << std::endl;

//

//                // 如果有物理量，读取物理量 (4字节，大端)

//                if (hasPhysicalValue) {

//                    if (pos + 3 >= length) break;

//

//                    // 1. 将4字节大端数据组合成uint32_t

//                    uint32_t physicalRaw = (static_cast<uint32_t>(data[pos]) << 24) |

//                        (static_cast<uint32_t>(data[pos + 1]) << 16) |

//                        (static_cast<uint32_t>(data[pos + 2]) << 8) |

//                        (static_cast<uint32_t>(data[pos + 3]));

//                    pos += 4;

//

//                    // 2. 安全的方式：使用 memcpy 进行类型转换

//                    std::memcpy(&sensor.physicalValue, &physicalRaw, sizeof(float));

//

//                    // 打印物理量原始数据[3](@ref)

//                    std::cout << "               物理量原始字节: 0x" << std::hex << std::uppercase

//                        << std::setw(2) << std::setfill('0') << (int)data[pos - 4] << " "

//                        << std::setw(2) << std::setfill('0') << (int)data[pos - 3] << " "

//                        << std::setw(2) << std::setfill('0') << (int)data[pos - 2] << " "

//                        << std::setw(2) << std::setfill('0') << (int)data[pos - 1]

//                        << std::dec << std::setfill(' ')

//                        << " | 物理量值: " << std::setprecision(4)

//                        << sensor.physicalValue << std::endl;

//                }

//

//                result.push_back(sensor);

//            }

//        }

//

//        std::cout << "=== 数据包解析完成，共解析 " << result.size() << " 个传感器 ===" << std::endl;

//    }

//    else {

//        std::cout << "!!! 无效的数据头 !!!" << std::endl;

//    }

//

//    return result;

//}

std::vector<SensorData> parseData(const uint8_t* data, size_t length) {

std::vector<SensorData> result;

// 检查最小长度和数据头

if (length < 5) return result; // 至少需要5字节(4字节头+1字节通道数)

// 检查数据头 "FBGV" 或 "FBGW"

if (data[0] == 0x46 && data[1] == 0x42 && data[2] == 0x47 &&

(data[3] == 0x56 || data[3] == 0x57)) {

bool hasPhysicalValue = (data[3] == 0x56); // 'V'表示有物理量

uint8_t channelCount = data[4]; // 通道数

size_t pos = 5; // 当前位置

std::cout << "=== 开始解析数据包 ===" << std::endl;

std::cout << "数据头: FBG" << (char)data[3] << std::endl;

std::cout << "通道数: " << (int)channelCount << std::endl;

std::cout << "数据总长度: " << length << " 字节" << std::endl;

std::cout << "字节序: 小端模式 (低位在前，高位在后)" << std::endl;

for (uint8_t ch = 0; ch < channelCount && pos < length; ch++) {

if (pos >= length) break;

uint8_t sensorCount = data[pos++]; // 当前通道的传感器数量

std::cout << "通道 " << (ch + 1) << " 传感器数量: " << (int)sensorCount << std::endl;

for (uint8_t s = 0; s < sensorCount && pos < length; s++) {

SensorData sensor;

sensor.channel = ch + 1; // 通道从1开始

sensor.sequence = s + 1; // 序列从1开始

sensor.hasPhysicalValue = hasPhysicalValue;

// 读取波长 (2字节，小端字节序 - 修正后)

if (pos + 1 >= length) break;

// 保存原始字节用于调试输出

uint8_t lowByte = data[pos];     // 低位字节（第一个字节）

uint8_t highByte = data[pos + 1]; // 高位字节（第二个字节）

// 修正：按照小端字节序解析，低位在前，高位在后

uint16_t wavelengthRaw = (static_cast<uint32_t>(highByte) << 8) |

(static_cast<uint32_t>(lowByte));

pos += 2;

sensor.wavelength = wavelengthRaw / 1000.0 + 1520.0; // 转换为nm

// 打印波长原始数据详细信息

std::cout << "通道 " << std::setw(2) << (int)sensor.channel

<< " 传感器 " << std::setw(2) << (int)sensor.sequence

<< " - 原始字节: 0x" << std::hex << std::uppercase

<< std::setw(2) << std::setfill('0') << (int)lowByte

<< " " << std::setw(2) << std::setfill('0') << (int)highByte

<< std::dec << std::setfill(' ')  // 恢复十进制输出

<< " | 字节序: 低(" << std::setw(3) << (int)lowByte

<< ") 高(" << std::setw(3) << (int)highByte << ")"

<< " | 原始值: " << std::setw(5) << wavelengthRaw

<< " | 计算波长: " << std::fixed << std::setprecision(3)

<< sensor.wavelength << " nm" << std::endl;

// 如果有物理量，读取物理量 (4字节，需要确认字节序)

if (hasPhysicalValue) {

if (pos + 3 >= length) break;

// 注意：这里需要确认物理量数据的字节序是否与波长一致

// 当前假设也是小端字节序

uint32_t physicalRaw = (static_cast<uint32_t>(data[pos + 3]) << 24) |

(static_cast<uint32_t>(data[pos + 2]) << 16) |

(static_cast<uint32_t>(data[pos + 1]) << 8) |

(static_cast<uint32_t>(data[pos]));

pos += 4;

// 使用 memcpy 进行类型转换

std::memcpy(&sensor.physicalValue, &physicalRaw, sizeof(float));

// 打印物理量原始数据

std::cout << "               物理量原始字节: 0x" << std::hex << std::uppercase

<< std::setw(2) << std::setfill('0') << (int)data[pos - 4] << " "

<< std::setw(2) << std::setfill('0') << (int)data[pos - 3] << " "

<< std::setw(2) << std::setfill('0') << (int)data[pos - 2] << " "

<< std::setw(2) << std::setfill('0') << (int)data[pos - 1]

<< std::dec << std::setfill(' ')

<< " | 物理量值: " << std::setprecision(4)

<< sensor.physicalValue << std::endl;

}

result.push_back(sensor);

}

}

std::cout << "=== 数据包解析完成，共解析 " << result.size() << " 个传感器 ===" << std::endl;

}

else {

std::cout << "!!! 无效的数据头 !!!" << std::endl;

}

return result;

}

boost::asio::io_context io_context;

boost::asio::ip::udp::socket socket;

std::thread receiveThread;

std::atomic<bool> receiving;

};

// FiberGratingAnalyzer 类实现

FiberGratingAnalyzer::FiberGratingAnalyzer() : pimpl(new Impl()) {}

FiberGratingAnalyzer::~FiberGratingAnalyzer() {

delete pimpl;

}

bool FiberGratingAnalyzer::setPort(int port) {

return pimpl->setPort(port);

}

bool FiberGratingAnalyzer::startReceiving(const DataCallback& callback) {

return pimpl->startReceiving(callback);

}

void FiberGratingAnalyzer::stopReceiving() {

pimpl->stop();

}

bool FiberGratingAnalyzer::isReceiving() const {

return pimpl->isReceiving();

}

// FiberGratingAnalyzer.h

#ifndef FIBER_GRATING_ANALYZER_H

#define FIBER_GRATING_ANALYZER_H

#include <string>

#include <functional>

#include <vector>

#include <cstdint>

// 前向声明

namespace boost::asio {

class io_context;

// class ip::udp;

}

namespace boost::asio::ip

{

class udp;

}

class FiberGratingAnalyzer {

public:

// 传感器数据结构

struct SensorData {

uint16_t channel;      // 通道号

uint8_t sequence;       // 序列号

double wavelength;      // 波长值 (nm)

float physicalValue;    // 物理量值 (如果有)

bool hasPhysicalValue; // 是否有物理量值

};

// 回调函数类型定义

using DataCallback = std::function<void(const std::vector<SensorData>&)>;

// 构造函数和析构函数

FiberGratingAnalyzer();

~FiberGratingAnalyzer();

// 设置接收数据的端口

bool setPort(int port);

// 开始接收数据

bool startReceiving(const DataCallback& callback);

// 停止接收数据

void stopReceiving();

// 检查是否正在接收数据

bool isReceiving() const;

private:

// 实现细节的PIMPL模式

class Impl;

Impl* pimpl;

};

#endif // FIBER_GRATING_ANALYZER_H

//FiberGratingAnalyzerUI.cpp

#include "udp/FiberGratingAnalyzerUI.h"

#include <iomanip>

#include <sstream>

#include <algorithm>

FiberGratingAnalyzerUI::FiberGratingAnalyzerUI()

: analyzer_(nullptr), isReceiving_(false) {

}

FiberGratingAnalyzerUI::~FiberGratingAnalyzerUI() {

StopReceiving();

}

void FiberGratingAnalyzerUI::SetAnalyzer(FiberGratingAnalyzer* analyzer) {

analyzer_ = analyzer;

}

void FiberGratingAnalyzerUI::ShowUI() {

if (!analyzer_) return;

ImGui::Begin(u8"光纤光栅分析器");

RenderControlPanel();

ImGui::Separator();

RenderDataDisplay();

ImGui::Separator();

RenderStatistics();

ImGui::Separator();

RenderCharts();

ImGui::End();

}

void FiberGratingAnalyzerUI::StartReceiving() {

if (isReceiving_ || !analyzer_) return;

// 设置端口 [1,6](@ref)

if (!analyzer_->setPort(8071)) {

ImGui::TextColored(ImVec4(1, 0, 0, 1), u8"端口设置失败!");

return;

}

isReceiving_ = true;

startTime_ = std::chrono::steady_clock::now();

totalPackets_ = 0;

totalSensors_ = 0;

// 清空历史数据

{

std::lock_guard<std::mutex> lock(dataMutex_);

currentData_.clear();

wavelengthHistory_.clear();

physicalValueHistory_.clear();

}

// 启动数据接收线程

dataThread_ = std::thread([this]() {

auto callback = [this](const std::vector<FiberGratingAnalyzer::SensorData>& sensors) {

this->UpdateData(sensors);

};

analyzer_->startReceiving(callback);

});

ImGui::TextColored(ImVec4(0, 1, 0, 1), u8"开始接收数据...");

}

void FiberGratingAnalyzerUI::StopReceiving() {

if (!isReceiving_ || !analyzer_) return;

isReceiving_ = false;

analyzer_->stopReceiving();

if (dataThread_.joinable()) {

dataThread_.join();

}

}

void FiberGratingAnalyzerUI::UpdateData(const std::vector<FiberGratingAnalyzer::SensorData>& sensors) {

std::lock_guard<std::mutex> lock(dataMutex_);

totalPackets_++;

totalSensors_ += sensors.size();

currentData_ = sensors;

// 更新历史数据用于图表显示

auto currentTime = std::chrono::steady_clock::now();

auto timePoint = std::chrono::duration_cast<std::chrono::milliseconds>(

currentTime - startTime_).count() / 1000.0f;

for (const auto& sensor : sensors) {

int sensorId = sensor.channel * 100 + sensor.sequence;

// 更新波长历史

wavelengthHistory_[sensorId].push_back(sensor.wavelength);

if (wavelengthHistory_[sensorId].size() > maxHistoryPoints_) {

wavelengthHistory_[sensorId].erase(wavelengthHistory_[sensorId].begin());

}

// 更新物理量历史（如果有）

if (sensor.hasPhysicalValue) {

physicalValueHistory_[sensorId].push_back(sensor.physicalValue);

if (physicalValueHistory_[sensorId].size() > maxHistoryPoints_) {

physicalValueHistory_[sensorId].erase(physicalValueHistory_[sensorId].begin());

}

}

}

}

void FiberGratingAnalyzerUI::RenderControlPanel() {

ImGui::Text(u8"控制面板");

if (!isReceiving_) {

if (ImGui::Button(u8"开始接收")) {

StartReceiving();

}

}

else {

if (ImGui::Button(u8"停止接收")) {

StopReceiving();

}

ImGui::SameLine();

ImGui::TextColored(ImVec4(0, 1, 0, 1), u8"运行中...");

}

ImGui::SameLine();

ImGui::Checkbox(u8"显示物理量", &showPhysicalValues_);

ImGui::SameLine();

ImGui::Checkbox(u8"显示图表", &showWavelengthCharts_);

ImGui::SameLine();

ImGui::Checkbox(u8"自动缩放", &autoScaleCharts_);

// 温度校准设置 [6](@ref)

if (ImGui::TreeNode(u8"温度校准设置")) {

ImGui::InputFloat(u8"温度系数 (nm/°C)", &tempCalibration_.coefficient);

ImGui::InputFloat(u8"参考波长 (nm)", &tempCalibration_.referenceWavelength);

ImGui::InputFloat(u8"参考温度 (°C)", &tempCalibration_.referenceTemperature);

ImGui::TreePop();

}

}

void FiberGratingAnalyzerUI::RenderDataDisplay() {

ImGui::Text(u8"实时数据");

std::lock_guard<std::mutex> lock(dataMutex_);

if (currentData_.empty()) {

ImGui::Text(u8"暂无数据");

return;

}

// 创建表格显示数据

if (ImGui::BeginTable(u8"SensorData", showPhysicalValues_ ? 5 : 4,

ImGuiTableFlags_Borders | ImGuiTableFlags_RowBg)) {

// 表头

ImGui::TableSetupColumn(u8"通道");

ImGui::TableSetupColumn(u8"序列号");

ImGui::TableSetupColumn(u8"波长 (nm)");

if (showPhysicalValues_) {

ImGui::TableSetupColumn(u8"物理量");

}

ImGui::TableSetupColumn(u8"状态");

ImGui::TableHeadersRow();

for (const auto& sensor : currentData_) {

ImGui::TableNextRow();

// 通道列

ImGui::TableSetColumnIndex(0);

ImGui::Text("%d", sensor.channel);

// 序列号列

ImGui::TableSetColumnIndex(1);

ImGui::Text("%d", sensor.sequence);

// 波长列

ImGui::TableSetColumnIndex(2);

ImGui::Text("%.3f", sensor.wavelength);

// 物理量列

if (showPhysicalValues_) {

ImGui::TableSetColumnIndex(3);

if (sensor.hasPhysicalValue) {

// 应用温度校准 [6](@ref)

float calibratedValue = sensor.physicalValue;

//ApplyTemperatureCalibration(const_cast<FiberGratingAnalyzer::SensorData&>(sensor));

ImGui::Text("%.4f", calibratedValue);

}

else {

ImGui::TextColored(ImVec4(0.5f, 0.5f, 0.5f, 1), "N/A");

}

}

// 状态列

int statusColumn = showPhysicalValues_ ? 4 : 3;

ImGui::TableSetColumnIndex(statusColumn);

// 根据波长值判断状态 [2](@ref)

if (sensor.wavelength < 1520.0 || sensor.wavelength > 1620.0) {

ImGui::TextColored(ImVec4(1, 0, 0, 1), u8"异常");

}

else {

ImGui::TextColored(ImVec4(0, 1, 0, 1), u8"正常");

}

}

ImGui::EndTable();

}

}

void FiberGratingAnalyzerUI::RenderStatistics() {

ImGui::Text(u8"统计信息");

auto currentTime = std::chrono::steady_clock::now();

auto elapsedSeconds = std::chrono::duration_cast<std::chrono::seconds>(

currentTime - startTime_).count();

ImGui::Text(u8"运行时间: %lld 秒", elapsedSeconds);

ImGui::Text(u8"接收数据包: %d", totalPackets_);

ImGui::Text(u8"传感器数据总数: %d", totalSensors_);

if (elapsedSeconds > 0) {

float packetRate = static_cast<float>(totalPackets_) / elapsedSeconds;

float sensorRate = static_cast<float>(totalSensors_) / elapsedSeconds;

ImGui::Text(u8"数据率: %.2f 包/秒, %.2f 传感器/秒", packetRate, sensorRate);

}

}

void FiberGratingAnalyzerUI::RenderCharts() {

if (!showWavelengthCharts_) return;

std::lock_guard<std::mutex> lock(dataMutex_);

if (wavelengthHistory_.empty()) return;

ImGui::Text(u8"波长变化趋势");

for (const auto& [sensorId, wavelengths] : wavelengthHistory_) {

if (wavelengths.empty()) continue;

int channel = sensorId / 100;

int sequence = sensorId % 100;

std::string label = u8"通道 " + std::to_string(channel) +

u8" - 传感器 " + std::to_string(sequence);

if (ImGui::TreeNode(label.c_str())) {

// 计算图表范围

float minVal = *std::min_element(wavelengths.begin(), wavelengths.end());

float maxVal = *std::max_element(wavelengths.begin(), wavelengths.end());

float range = maxVal - minVal;

if (range < 0.1f) range = 0.1f; // 最小范围

ImVec2 graphSize(-1, 100);

// 波长图表

ImGui::Text(u8"波长 (nm): %.3f", wavelengths.back());

if (ImGui::BeginChild(("WaveChart_" + std::to_string(sensorId)).c_str(),

graphSize, false)) {

ImGui::PlotLines("", wavelengths.data(), wavelengths.size(),

0, nullptr, minVal - range * 0.1f,

maxVal + range * 0.1f, graphSize);

}

ImGui::EndChild();

// 物理量图表（如果有）

if (showPhysicalValues_ && physicalValueHistory_.count(sensorId)) {

const auto& physicalValues = physicalValueHistory_.at(sensorId);

if (!physicalValues.empty()) {

ImGui::Text(u8"物理量: %.4f", physicalValues.back());

if (ImGui::BeginChild(("PhysChart_" + std::to_string(sensorId)).c_str(),

graphSize, false)) {

float pMin = *std::min_element(physicalValues.begin(), physicalValues.end());

float pMax = *std::max_element(physicalValues.begin(), physicalValues.end());

float pRange = pMax - pMin;

if (pRange < 0.1f) pRange = 0.1f;

ImGui::PlotLines("", physicalValues.data(), physicalValues.size(),

0, nullptr, pMin - pRange * 0.1f,

pMax + pRange * 0.1f, graphSize);

}

ImGui::EndChild();

}

}

ImGui::TreePop();

}

}

}

void FiberGratingAnalyzerUI::ApplyTemperatureCalibration(FiberGratingAnalyzer::SensorData& sensor) {

if (sensor.hasPhysicalValue) {

// 基于布拉格光栅温度特性进行校准 [6](@ref)

float wavelengthShift = sensor.wavelength - tempCalibration_.referenceWavelength;

float temperatureChange = wavelengthShift / tempCalibration_.coefficient;

sensor.physicalValue = tempCalibration_.referenceTemperature + temperatureChange;

}

} //FibreGratingAnalyzerUI.h

#ifndef FIBER_GRATING_ANALYZER_UI_H

#define FIBER_GRATING_ANALYZER_UI_H

#include "udp/FiberGratingAnalyzer.h"

#include <imgui.h>

#include <vector>

#include <map>

#include <atomic>

#include <mutex>

#include <thread>

#include <chrono>

class FiberGratingAnalyzerUI {

public:

FiberGratingAnalyzerUI();

~FiberGratingAnalyzerUI();

void ShowUI();

void SetAnalyzer(FiberGratingAnalyzer* analyzer);

private:

void StartReceiving();

void StopReceiving();

void UpdateData(const std::vector<FiberGratingAnalyzer::SensorData>& sensors);

void RenderControlPanel();

void RenderDataDisplay();

void RenderStatistics();

void RenderCharts();

void ApplyTemperatureCalibration(FiberGratingAnalyzer::SensorData& sensor);

private:

FiberGratingAnalyzer* analyzer_;

std::atomic<bool> isReceiving_;

std::thread dataThread_;

mutable std::mutex dataMutex_;

// 实时数据存储

std::vector<FiberGratingAnalyzer::SensorData> currentData_;

std::map<int, std::vector<float>> wavelengthHistory_;

std::map<int, std::vector<float>> physicalValueHistory_;

// 显示设置

int maxHistoryPoints_ = 100;

bool showPhysicalValues_ = true;

bool showWavelengthCharts_ = true;

bool autoScaleCharts_ = true;

// 统计信息

int totalPackets_ = 0;

int totalSensors_ = 0;

std::chrono::steady_clock::time_point startTime_;

// 温度校准参数 [6](@ref)

struct TemperatureCalibration {

float coefficient = 0.01f; // 温度系数 (nm/°C)

float referenceWavelength = 1550.0f; // 参考波长 (nm)

float referenceTemperature = 25.0f; // 参考温度 (°C)

} tempCalibration_;

};

#endif // FIBER_GRATING_ANALYZER_UI_H

//FibreGratingManager.h

#pragma once

#include <iostream>

#include <thread>

#include <atomic>

#include <mutex>

#include <condition_variable>

#include <chrono>

#include <iomanip>

#include<algorithm>

#include "data/Data.h"

#include"FiberGratingAnalyzer.h"

class FibreGratingManager {

private:

std::atomic<bool> running_{ false };

std::atomic<bool> dataReceiving_{ false };

std::thread workerThread_;

std::mutex dataMutex_;

std::condition_variable cv_;

// 统计变量

std::atomic<int> totalPackets_{ 0 };

std::atomic<int> totalSensors_{ 0 };

std::chrono::steady_clock::time_point startTime_;

public:

FibreGratingManager() = default;

~FibreGratingManager() {

stop();

}

// 启动光纤光栅分析器（非阻塞）

bool start() {

if (dataReceiving_.load()) {

//std::cout << "光纤光栅分析器已在运行中..." << std::endl;

return false;

}

// 重置统计信息

totalPackets_ = 0;

totalSensors_ = 0;

startTime_ = std::chrono::steady_clock::now();

running_ = true;

dataReceiving_ = true;

// 在独立线程中启动数据接收

workerThread_ = std::thread([this]() {

this->runAnalyzer();

});

std::cout << "光纤光栅分析器启动成功" << std::endl;

return true;

}

// 停止光纤光栅分析器

void stop() {

if (!dataReceiving_.load()) return;

running_ = false;

dataReceiving_ = false;

// 通知等待的线程

cv_.notify_all();

if (workerThread_.joinable()) {

workerThread_.join();

}

displayFinalStatistics();

}

// 检查是否正在运行

bool isRunning() const {

return dataReceiving_.load();

}

// 获取运行统计信息

struct Statistics {

int totalPackets;

int totalSensors;

long long runTimeSeconds;

double packetsPerSecond;

double sensorsPerSecond;

};

Statistics getStatistics() const {

auto currentTime = std::chrono::steady_clock::now();

auto runTime = std::chrono::duration_cast<std::chrono::seconds>(

currentTime - startTime_).count();

Statistics stats{};

stats.totalPackets = totalPackets_.load();

stats.totalSensors = totalSensors_.load();

stats.runTimeSeconds = runTime;

stats.packetsPerSecond = (runTime > 0) ?

static_cast<double>(stats.totalPackets) / runTime : 0.0;

stats.sensorsPerSecond = (runTime > 0) ?

static_cast<double>(stats.totalSensors) / runTime : 0.0;

return stats;

}

private:

void runAnalyzer() {

const int LISTEN_PORT = 8071;

const std::string LOCAL_ADDRESS = "0.0.0.0";

std::cout << "=== 光纤光栅解调仪数据接收程序 ===" << std::endl;

std::cout << "监听地址: " << LOCAL_ADDRESS << std::endl;

std::cout << "监听端口: " << LISTEN_PORT << std::endl;

std::cout << "开始时间: " << __DATE__ << " " << __TIME__ << std::endl;

std::cout << "==================================" << std::endl << std::endl;

FiberGratingAnalyzer analyzer;

// 设置端口

if (!analyzer.setPort(LISTEN_PORT)) {

std::cerr << "错误: 无法在端口 " << LISTEN_PORT << " 上启动监听" << std::endl;

dataReceiving_ = false;

return;

}

// 数据回调函数

auto dataCallback = [this](const std::vector<FiberGratingAnalyzer::SensorData>& sensors) {

this->processSensorData(sensors);

};

// 开始接收数据[1,2](@ref)

if (!analyzer.startReceiving(dataCallback)) {

std::cerr << "错误: 无法启动数据接收线程" << std::endl;

dataReceiving_ = false;

return;

}

std::cout << "数据接收已启动，开始监听..." << std::endl;

// 接收循环[2,4](@ref)

try {

while (running_.load()) {

std::unique_lock<std::mutex> lock(dataMutex_);

cv_.wait_for(lock, std::chrono::milliseconds(100), [this]() {

return !running_.load();

});

// 心跳检测

static int heartbeat = 0;

if (++heartbeat % 10 == 0 && totalPackets_.load() == 0) {

std::cout << "等待数据输入中..." << std::endl;

}

}

}

catch (const std::exception& e) {

std::cerr << "数据接收异常: " << e.what() << std::endl;

}

// 停止接收[1](@ref)

analyzer.stopReceiving();

dataReceiving_ = false;

}

void processSensorData(const std::vector<FiberGratingAnalyzer::SensorData>& sensors) {

totalPackets_++;

totalSensors_ += sensors.size();

auto currentTime = std::chrono::steady_clock::now();

auto elapsed = std::chrono::duration_cast<std::chrono::seconds>(

currentTime - startTime_).count();

// 实时显示统计信息[2](@ref)

std::cout << "\033[2J\033[1;1H";

std::cout << "=== 实时数据监控 ===" << std::endl;

std::cout << "运行时间: " << elapsed << " 秒" << std::endl;

std::cout << "数据包数: " << totalPackets_.load() << std::endl;

std::cout << "传感器数据总数: " << totalSensors_.load() << std::endl;

std::cout << "平均频率: " << (elapsed > 0 ? totalPackets_.load() / elapsed : 0) << " Hz" << std::endl;

std::cout << "==========================" << std::endl;

if (!sensors.empty()) {

std::cout << "最新数据包详情 (" << sensors.size() << " 个传感器):" << std::endl;

std::cout << std::setw(6) << "通道"

<< std::setw(8) << "序列号"

<< std::setw(12) << "波长(nm)"

<< std::setw(15) << "物理量"

<< std::setw(10) << "状态" << std::endl;

std::cout << std::string(55, '-') << std::endl;

for (const auto& sensor : sensors) {

std::cout << std::setw(6) << sensor.channel

<< std::setw(8) << sensor.sequence

<< std::setw(12) << std::fixed << std::setprecision(3) << sensor.wavelength;

if (sensor.hasPhysicalValue) {

std::cout << std::setw(15) << std::setprecision(4) << sensor.physicalValue;

}

else {

std::cout << std::setw(15) << "N/A";

}

if (sensor.wavelength < 1520.0 || sensor.wavelength > 1620.0) {

std::cout << std::setw(10) << "异常";

}

else {

std::cout << std::setw(10) << "正常";

}

std::cout << std::endl;

std::lock_guard<std::mutex> lock(g_sensorDataMutex);

g_sensorDataCollection.insert(g_sensorDataCollection.end(), sensors.begin(), sensors.end());

// 打印最近添加的数据（可选：只打印最后几条）

int showCount = 1; // 显示最近3条或更少

std::cout << "最近添加的 " << showCount << " 条数据:" << std::endl;

for (int i = sensors.size() - showCount; i < sensors.size(); ++i) {

const auto& sensor = sensors[i];

std::cout << "  通道:" << sensor.channel

<< " 序列号:" << static_cast<int>(sensor.sequence)

<< " 波长:" << std::fixed << std::setprecision(3) << sensor.wavelength << "nm";

if (sensor.hasPhysicalValue) {

std::cout << " 物理量:" << std::setprecision(4) << sensor.physicalValue;

}

else {

std::cout << " 物理量:N/A";

}

std::cout << std::endl;

}

std::cout << "==========================" << std::endl;

}

}

std::cout << std::endl << "正在接收数据... 使用 stop() 方法停止";

std::cout.flush();

}

void displayFinalStatistics() {

auto endTime = std::chrono::steady_clock::now();

auto totalDuration = std::chrono::duration_cast<std::chrono::seconds>(

endTime - startTime_).count();

std::cout << "\n=== 接收统计 ===" << std::endl;

std::cout << "总运行时间: " << totalDuration << " 秒" << std::endl;

std::cout << "总数据包数: " << totalPackets_.load() << std::endl;

std::cout << "总传感器数据: " << totalSensors_.load() << std::endl;

if (totalDuration > 0) {

std::cout << "平均数据率: " << (totalPackets_.load() / totalDuration) << " 包/秒" << std::endl;

std::cout << "平均传感器数据率: " << (totalSensors_.load() / totalDuration) << " 数据/秒" << std::endl;

}

std::cout << "光纤光栅分析器已停止" << std::endl;

}

};